Brian Dent Limited - Reversing & Manoeuvring Policy

The Purpose of this document: To establish a safe set of instructions or policy and procedures for vehicle reversing & manoeuvring within the day-to-day use of commercial vehicles.

The Aim:

The aim of this document is to prevent incidents and accidents caused during the operation of commercial vehicles by failure to:

- Use rear view mirrors,

- Perform circle checks,

- Physically face in the direction of reversing when possible

- Utilise a competent banksman.

Responsibility:

All employees and Subcontractors engaged by Brian Dent Limited

Policy:

Drivers must be particularity careful when reversing any vehicle, plant or equipment. If the driver has to reverse, he/she must follow the listed procedures and, whenever possible, use a competent banksman wearing appropriate Hi-Viz clothing and protective footwear, guide or other person to assist the driver in reversing. The driver is ultimately responsible for the safe operation of the vehicle.

If either party is unable to converse in the same language as the other, then other personnel should be sought. Failing this a signalling procedure should be used. There are industry standard signals, which should be adopted.

Always obey the site rules with regards to traffic directional flow and speed limits.

Procedures:

- Avoid reversing – Whenever possible, position your vehicle so you can drive forward rather than reverse.

- If a vehicle cannot be positioned so it can drive forward into and out of a parked position, it must be backed in with a banksman so it will not have to back into traffic later.

- A clear and agreed signalling procedure must be agreed between the driver & the banksman before commencing any reversing manoeuvres.

- Parking – Always attempt to park in a location and in a manner that avoids reversing.  If reversing is required, it is preferable to reverse into parking spaces to avoid reversing out.

- Walk around vehicle (Circle Check) – Upon approaching a parked vehicle, the driver must “walk around the vehicle” prior to moving the unit to ensure it is free of obstructions or hazards & pedestrians. Please take the time to look around. It will be well worth the effort. If you do not move promptly after the check, please recheck the surrounding area again as obstructions may have reoccurred.

- Check to ensure all reversing lights and audible warnings are functioning properly.

- Use a Banksman – It is up to the driver to seek the services of a responsible and competent person to be a banksman.

- The Banksman must be in a position where he/she can see the driver’s blind spot.

- The banksman must remain outside of the vehicle and direct the driver safely and to negotiate obstacles until the vehicle as finished reversing or manoeuvring.

- The driver must obey the banksman’s directions but use reasonable precaution and his/her own good judgment to prevent incidents/accidents.

Important:

- If the driver loses sight of the banksman during before or during a manoeuvre, he/she should stop immediately, apply the handbrake & await sight of the banksman. If the banksman does not appear in due course, switch off the engine and investigate. Always wear a high-viz jacket and protective footwear when exiting your vehicle on a site.

Remember:

It is an offence under section 106 of the road vehicles (Construction & Use Regulations) 1986 to reverse your vehicle further than necessary.

If in doubt, STOP the vehicle and check for hazards.

Be glad you stopped ….

Don’t wish you’d stopped.

I acknowledge receipt of the guidance document and promise to adhere to the directions during the course of my employment at Brian Dent Limited

Sign ___________________________________

Print ___________________________________                 Date ___________________________